Campus Network Best Practices:

Hints on Network Diagrams

Cambodian Research and Education Network

.

Network Diagram Hints

•    Use separate diagrams for separate layers

–  Physical (cabling)

–  Switching (layer 2)

–  Routing (layer 3)

–  Frequently can combine Switching/Routing

•  Helpful to have the physical diagram be an overlay on your campus map to show relations and distances between buildings

•  If you combine layers 2/3, make it clear where you are switching and where you are routing

Sample Physical Diagram

|  | 2x |  | 8x62.5 fiber |
| --- | --- | --- | --- |
| 2.4Ghz 54Mbs | 62.5 fiber |  |  |
| wireless |  |  | 12xG.652.D fiber |
|  |  |  | Splice |
|  |  | 4xG.652.D fiber | 4xG.652.D fiber |
|  | 2x |  |  |
|  | cat5e |  |  |

Sample Layer 3 Diagram

ISP

Cisco 2901	Cisco 3750-12D

Border Router (no NAT)	Core Router

62.63.64.32/29      128.223.2.64/29

128.223.10.0/24

128.223.20.0/24	Art Faculty

Server Network

128.223.12.0/23

128.223.14.0/24

128.223.16.0/24

Admin Building	Social Science	General Science Faculty

Faculty

Sample Layer 2/3 Diagram

Cisco 2901	Cisco 3750-12D

Border Router (no NAT)	Core Router

62.63.64.32/29      128.223.2.64/29

ISP

128.223.10.0/24

128.223.20.0/24

Server Network

128.223.12.0/23

128.223.14.0/24

Art Faculty

128.223.16.0/24

General Science Faculty

Admin Building	Social Science Faculty

Questions?