Network Management &

Monitoring

Netflow

NetFlow Overview

These materials are licensed under the Creative Commons Attribution-Noncommercial 3.0 Unported license (http://creativecommons.org/licenses/by-nc/3.0/)

Agenda

Netflow

–   What it is and how it works

–   Uses and Applications

Flow-tools

–   Architectural issues

–   Software, tools etc

Lab

Network Flows

•  Packets or frames that have a common attribute.

•  Creation and expiration policy – what conditions start and stop a flow.

•    Counters – packets, bytes, time.

•  Routing information – AS, network mask, interfaces.

Cisco’s Definition of a Flow

Unidirectional sequence of packets sharing

- Source IP address

- Destination IP address

- Source port for UDP or TCP, 0 for other protocols

- Destination port for UDP or TCP, type and code for ICMP, or 0 for other protocols

- IP protocol

- Ingress interface (SNMP ifIndex)

- IP Type of Service

Network Flows

•    Unidirectional or bidirectional.

•  Bidirectional flows can contain other information such as round trip time, TCP behavior.

•  Application flows look past the headers to classify packets by their contents.

•    Aggregated flows – flows of flows.

Working with Flows

•  Generate the flows from device (usually a router)

•  Export flows from the device to collector

–  Configure version of flows –  Sampling rates

•  Collect the flows

–  Tools to Collect Flows - Flow-tools

•  Analyze them

–  More tools available, can write your own

Flow Descriptors

•  A Key with more elements will generate more flows.

•    Greater number of flows equals:

–      More post processing time to generate reports

–  more memory and CPU requirements for device generating flows

–  More storage needed on the flow processing server

•  Depends on application. Traffic engineering vs. intrusion detection.

Flow Accounting

•  Accounting information accumulated with flows.

•    Packets, Bytes, Start Time, End Time.

•  Network routing information – masks and autonomous system number.

Flow Generation/Collection

Passive monitor

•  A passive monitor (usually a Unix host) receives all data and generates flows.

•    Resource intensive

Router or other existing network device

•  Router or other existing devices like switch, generate flows.

•    Sampling is possible

•    Nothing new needed

Passive Monitor Collection

Workstation A	Workstation B

Flow probe connected

to switch port in Campus “ traffic mirror” mode

Passive Collector

•  Using passive collection, not all flows in the network will be seen as opposed to collection from the router

•  The collector will only see flows from the network point it is connected on

•     However this method does relieve the router from processing netflows and exporting them

•  Useful on links with only one entry into the network or where only flows from one section of the network are needed

Router Collection

LAN	LAN

| LAN | LAN |
| --- | --- |

Internet

Flow collector

stores exported flows from router.

Router Collection

•  With this method, all flows in the network can be observed

•  However, more work for the router in processing and exporting the flows

•  Optionally, one can choose on which interfaces netflow collection is needed and not activate it on others

•  Also, if there is a router on each LAN, netflow can be activated on those routers to reduce the load on the core router

Cisco NetFlow

•    Unidirectional flows.

•    IPv4 unicast and multicast.

•    Aggregated and unaggregated.

•    Flows exported via UDP.

•    Supported on IOS and CatOS platforms.

•    Catalyst NetFlow is different implementation.

Cisco NetFlow Versions

•    4 Unaggregated types (1,5,6,7).

•    14 Aggregated types (8.x, 9).

•    Each version has its own packet format.

•    Version 1 does not have sequence numbers

– no way to detect lost flows.

•  The “version” defines what type of data is in the flow.

•    Some versions specific to Catalyst platform.

NetFlow Version 1

•  Key fields: Source/Destination IP, Source/Destination Port, IP Protocol, ToS, Input interface.

•  Accounting: Packets, Octets, Start/End time, Output interface

•    Other: Bitwise OR of TCP flags.

•    Obsolete

NetFlow Versions 2-4

•    Cisco internal

•    Were never released

NetFlow v5

•  Key fields: Source/Destination IP, Source/Destination Port, IP Protocol, ToS, Input interface.

•  Accounting: Packets, Octets, Start/End time, Output interface.

•  Other: Bitwise OR of TCP flags, Source/Destination AS and IP Mask.

•  Packet format adds sequence numbers for detecting lost exports.

•    IPv4 only

NetFlow v8

•    Aggregated v5 flows.

•  Not all flow types available on all equipments

•  Much less data to post process, but loses fine granularity of v5 – no IP addresses.

NetFlow v9

•    IPv6 support

•    Additional fields like MPLS labels

•    Builds on earlier versions

Cisco IOS Configuration

•    Configured on each input interface.

•    Define the version.

•  Define the IP address of the collector (where to send the flows).

•    Optionally enable aggregation tables.

•  Optionally configure flow timeout and main (v5) flow table size.

•    Optionally configure sample rate.

Cisco IOS Configuration

ip flow-top-talkers

top 10

sort-by bytes

| gw-169-223-2-0#sh ip flow top-talkers |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SrcIf | SrcIPaddress | DstIf | DstIPaddress | Pr SrcP DstP Bytes |  |  |  |
| Fa0/1 | 169.223.2.2 | Fa0/0 | 169.223.11.33 | 06 | 0050 | 0B64 | 3444K |
| Fa0/1 | 169.223.2.2 | Fa0/0 | 169.223.11.33 | 06 | 0050 | 0B12 | 3181K |
| Fa0/0 | 169.223.11.33 | Fa0/1 | 169.223.2.2 | 06 | 0B12 | 0050 | 56K |
| Fa0/0 | 169.223.11.33 | Fa0/1 | 169.223.2.2 | 06 | 0B64 | 0050 | 55K |
| Fa0/1 | 169.223.2.2 | Local | 169.223.2.1 | 01 | 0000 | 0303 | 18K |
| Fa0/1 | 169.223.2.130 | Fa0/0 | 64.18.197.134 | 06 | 9C45 | 0050 | 15K |
| Fa0/1 | 169.223.2.130 | Fa0/0 | 64.18.197.134 | 06 | 9C44 | 0050 | 12K |
| Fa0/0 | 213.144.138.195 | Fa0/1 | 169.223.2.130 | 06 | 01BB DC31 |  | 7167 |
| Fa0/0 | 169.223.15.102 | Fa0/1 | 169.223.2.2 | 06 | C917 | 0016 | 2736 |
| Fa0/1 | 169.223.2.2 | Local | 169.223.2.1 | 06 | DB27 0016 |  | 2304 |

10 of 10 top talkers shown. 49 flows processed.

Cisco Command Summary

•  Enable CEF (done by default)

–   ip cef

•  Enable flow on each interface

ip route cache flow

OR

ip flow ingress

ip flow egress

•  View flows

–   show ip cache flow

–   show ip flow top-talkers

Cisco Command Summary

• 	Exporting Flows to a collector

ip flow-export version 5 [origin-as|peer-as] ip flow-export destination x.x.x.x <udp-port>

• 	Origin AS will include the origin AS Number in the flow while Peer AS will only include the AS Number of the peering neighbor

• 	Exporting aggregated flows

ip flow-aggregation cache as|prefix|dest|source|proto enabled

export destination x.x.x.x <udp-port>

Flows and Applications

Uses for NetFlow

•   Problem identification / solving

–   Traffic classification

–  DoS Traceback (some slides by Danny McPherson)‏

•   Traffic Analysis and Engineering

–   Inter-AS traffic analysis

–   Reporting on application proxies

•   Accounting (or billing)

–  Cross verification from other sources –  Can cross-check with SNMP data

Detect Anomalous Events: SQL “Slammer” Worm*

Flow-based Detection (cont)*

Once baselines are built anomalous activity can be detected

–  Pure rate-based (pps or bps) anomalies may be legitimate or

malicious

–  Many misuse attacks can be immediately recognized, even without baselines (e.g., TCP SYN or RST floods)‏

–  Signatures can also be defined to identify “interesting” transactional data (e.g., proto udp and port 1434 and 404 octets(376 payload) == slammer!)‏

–  Temporal compound signatures can be defined to detect with higher precision

Flow-based Commercial Tools…*

Commercial Detection: A Large Scale DOS Attack

Accounting

Flow based accounting can be a good supplement to SNMP based accounting.

References

•      flow-tools:

http://www.splintered.net/sw/flow-tools

•     WikiPedia:

http://en.wikipedia.org/wiki/Netflow

•      NetFlow Applications

http://www.inmon.com/technology/netflowapps.php

•      Netflow HOW-TO

http://www.linuxgeek.org/netflow-howto.php

•      IETF standards effort:

http://www.ietf.org/html.charters/ipfix-charter.html

References

•      Abilene NetFlow page

http://abilene-netflow.itec.oar.net/

•      Flow-tools mailing list:

flow-tools@splintered.net

•      Cisco Centric Open Source Community

http://cosi-nms.sourceforge.net/related.html

•      Cisco NetFlow Collector User Guide

http://www.cisco.com/en/US/docs/net_mgmt/netflow_collection_engine/6.0/tier_one/ user/guide/user.html